CH-C-0015-0201                     * CH-C-0015-0201 is a Hall-Effect current transducer.

IPN = 15 A

Current Transducer                              * It can be used for the detection of DC, AC, pulse.

Applications

- AC variable speed drives and servo motor drives

- Static converters for DC motor drives

- Battery supplied applications

- Uninterruptible Power Supplies(UPS)

- Switched Mode Power Supplies (SMPS)

- Power supplies for welding applications.

Advantages

- Excellent accuracy

- Very good linearity

- Low temperature drift

Special features

- Printed circuit board mounting.

- Unipolar voltage supply.

Features

- Closed loop (compensated) current transducer using the Hall-Effect

| # Electrical data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPN | Primary nominal r.m.s current |  |  |  |  | 15 |  | A |  |
| IP | Primary current, measuring range |  |  |  |  | 0… 45① |  |  | A |
| Vout | Analog output voltage @ Ip Ip=0 |  |  |  |  | 2.5±(0.625·IP / IPN) |  |  | V |
|  | Ip=0 |  |  |  |  | 2.5±0.025② 2.5 |  |  | V |
| Ns | Number of secondary turns (0.1%) |  |  |  |  | 2000 |  |  |  |
| RL | Load resistance |  |  |  |  | ≥2 | k |  |  |
| VC | Supply voltage ( 5%) |  |  |  |  | 5 |  | V |  |
| IC | Idle Current consumption |  |  |  |  | 20 |  | mA |  |
| Vd | AC isolation test, 50 Hz, 1min |  |  |  |  | 3 |  | kV |  |

| # Accuracy – Dynamic performance data |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | Accuracy @ IPN , TA = 25C | 5V ( 5%) |  |  |  |  0.20 |  |  |  | % |
| L | Linearity error |  |  |  |  | < 0.15 |  |  |  | % |
|  | Offset current @ IP = 0, TA = 25C @100 Amax 0 0 |  |  |  |  |  0.60 |  |  |  | % |
| VOM | Residual current @ IP = 0, after an overload of 3  IPN 0 |  |  |  |  |  0.4 | mV |  |  |  |
| IOT | Thermal drift of IO @100 Amax | (0℃…+100℃) |  |  |  |  0.35 |  |  |  | mA |
| tra | Response time @ 10% of IPN |  |  |  |  | <200 |  |  |  | ns |
| di/dt | di/dt accurately followed |  |  |  |  | >100 |  |  | A/s |  |
| f | Frequency bandwidth (-1 dB) |  |  |  |  | DC…100 |  |  | kHz |  |

| # General data |  |  |  |  |
| --- | --- | --- | --- | --- |
| TA | Ambient operating temperature |  | -10..+100 | C |
| m | Mass |  | 12 | g |

Notes:  ①Only in refout mode or  with external REF less than 2.525V and greater than 2.475V.

② Absolute value @ TA = 25C  ,2.475< VOUT <2.525

③The primary winding is with 3 wires parallel.

## Remarks

- VOUT is positive when Ip flows from terminals 1,2,3 to terminals 4,5,6

- This is a standard model .For different versions (supply voltages,turns ratios,unidirectional

measurements….), please contact us.