CH-C-0025-0201                    * CH-C-0025-0201 is a Hall-Effect current transducer.

IPN = 25 A

Current Transducer                              * It can be used for the detection of DC, AC, pulse.

Applications

- AC variable speed drives and servo motor drives

- Static converters for DC motor drives

- Battery supplied applications

- Uninterruptible Power Supplies(UPS)

- Switched Mode Power Supplies (SMPS)

- Power supplies for welding applications.

Advantages

- Excellent accuracy

- Very good linearity

- Low temperature drift

Special features

- Printed circuit board mounting.

- Unipolar voltage supply.

Features

- Closed loop (compensated) current transducer using the Hall-Effect

| # Electrical data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IPN | Primary nominal r.m.s current |  |  |  |  |  |  |  | 25 | A |  |  |  |  |  |
| IP | Primary current, measuring range |  |  |  |  |  |  |  | 0… 75① | A |  |  |  |  |  |
| Vout |  | Analog output voltage @ Ip Ip=0 |  |  | 2.5±(0.625·IP / IPN) |  |  |  |  |  |  |  |  | V |  |
|  |  | Ip=0 |  |  | x | 2.5±0.025② |  |  |  |  |  |  |  | V |  |
| Ns |  | Number of secondary turns (0.1%) |  |  | 2000 |  |  |  |  |  |  |  |  |  |  |
| RL |  | Load resistance |  |  | ≥2 |  |  |  |  |  |  |  |  |  | k |
| VC |  | Supply voltage ( 5%) |  |  |  |  |  | 5 |  |  |  | V |  |  |  |
| IC |  | Current consumption @ VC =5V |  |  |  |  |  | 20 |  |  |  | mA |  |  |  |
| Vd |  | r.m.s voltage for AC isolation test, 50 Hz, 1min |  |  |  |  | 3 |  |  |  | kV |  |  |  |  |

| # Accuracy – Dynamic performance data |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X | Accuracy @ IPN , TA = 25C |  | 5V ( 5%) |  |  0.20 |  |  | % |
|  | Accuracy with RIM @ IPN,TA=25C |  |  |  |  0.60 | % |  |  |
| L | Linearity error |  |  |  | < 0.15 |  |  | % |
| TCVOUT |  | Thermal drift of VOUT / VREF @ IP=0 -10C..+100C |  |  | 50 |  | Ppm/K |  |
| TCVG |  | Thermal drift of the gain -10C..+100C |  |  | 50 |  | Ppm/K |  |
| VOM |  | Residual voltage @ Ip=0,after an overload of 3 x IPN |  |  |  0.4 |  | mV |  |
| tr |  | Response time @ 90% of IPN |  |  | <200 |  | ns |  |
| di/dt |  | di/dt accurately followed |  |  | >100 |  | A/s |  |
| f |  | Frequency bandwidth(-1dB) |  |  | DC..200 |  | kHz |  |

| # General data |  |  |  |  |
| --- | --- | --- | --- | --- |
| TA | Ambient operating temperature |  | -10..+100 | C |
| m | Mass |  | 12 | g |

Notes: ① Only in refout mode or with external REF  less than 2.525V and greater than 2.475V .;② VOUT is linked to VREF ,by conception the difference between these two nodes for IP=0 is maximum 25mV , 2.475<VOUT<2.525V  ③The primary winding is with 3 wires parallel.

Remarks

- VOUT is positive when Ip flows from terminals 1,2,3 to terminals 4,5,6

- This is a standard model .For different versions (supply voltages,turns ratios,unidirectional

measurements….), please contact us.