Current Transducer CH-C-0015-0203                  IPN = 50At

* CH-C-0050-0203 is a Hall-Effect current transducer.

* It can be used for the detection of DC, AC, pulse.

| ## Electrical data |  |  |  |
| --- | --- | --- | --- |
| IPN | Primary nominal r.m.s current | 50 | At |
| IP | Primary current, measuring range | 0… 150① | At |
| Vout | Analog output voltage @ Ip Ip=0 | 2.5±(0.625·IP / IPN) | V |
|  | Ip=0 | 2.5±0.025② 2.5 | V |
| Ns | Number of secondary turns (0.1%) | 1400 |  |
| RL | Load resistance | ≥2 | k |
| VC | Supply voltage ( 5%) | 5 | V |
| IC | Idle current consumption @ VC =5V | 20 | mA |
| Vd | AC isolation test, 50 Hz, 1min | 3 | kV |

| Accuracy – Dynamic performance data |  |  |  |
| --- | --- | --- | --- |
| X | Accuracy @ IPN , TA = 25C 5V ( 5%) f=1kHz |  0.70 | % |
|  | Accuracy with RIM @ IPN,TA=25C |  0.60 | % |
| εL | Linearity error | <0.1 | % |
| VOM | Residual voltage @ Ip=0,after an overload of 3 x IPN |  2.5 | mV |
| tra | Reaction time @ 10% of IPN | <200 | ns |
| di/dt | di/dt accurately followed | >100 | A/s |
| f | Frequency bandwidth(-1dB) | DC…200 | kHz |

| ## General data |  |  |  |
| --- | --- | --- | --- |
| TA | Ambient operating temperature | -10..+100 | C |
| M | Mass | 12 | g |

Notes:  ①Only in refout mode or  with external REF less than 2.525V and greater than 2.475V.

② Absolute value @ TA = 25C  ,2.475< VOUT <2.525

③ The primary winding is with 3 wires parallel.

Features

- Closed loop (compensated) current transducer using the Hall-Effect

Advantages

- Excellent accuracy

- Very good linearity

- Low temperature drift

Applications

- AC variable speed drives and servo motor drives

- Static converters for DC motor drives

- Battery supplied applications

- Uninterruptible Power Supplies (UPS)

- Switched Mode Power Supplies(SMPS)

- Power supplies for welding applications.

Special features

- Printed circuit board mounting.

- Unipolar voltage supply.

Mechanical Characteristic:  General tolerance  ±0.1mm

### Remarks

- Vout is positive when IP flows from terminals 1,2,3 to terminals 6,5,4.

- This is a standard model .For different versions (supply voltages,turns ratios,unidirectional

measurements….), please contact us.